# Edge Board Deployment: Ubuntu 22.04 + ROS 2 Humble + MoveIt2

Recommended setup path for the dual-arm box-opening system

# Purpose

This document describes the recommended deployment path for running ROS 2 Humble, MoveIt2, perception nodes, and dual-arm control on an edge compute board connected to robot arms, grippers, and RGB-D cameras.

# Base System

Install the Ubuntu 22.04 based board image using the matching board flashing guide.

Verify first boot, storage availability, network connectivity, and system time synchronization.

Install terminal utilities, development tools, browser support, camera tools, and Python package tooling for the board architecture.

Keep model checkpoints and datasets outside the repository, then reference them through ROS parameters or command-line arguments.

# ROS 2 Humble

Set the locale to UTF-8.

Enable the Ubuntu Universe repository.

Install curl, gnupg, lsb-release, and other repository-management tools.

Add the ROS 2 package key and source list.

Update package metadata.

Install either the desktop profile, the full desktop profile, or the ROS-Base profile depending on available disk space and whether RViz is needed.

Install development tools such as colcon, vcstool, and rosdep.

Source the ROS setup file and verify installation with the talker-listener demo.

# MoveIt2

Binary installation is the simplest path for robot integration. Source installation is useful when local patches, tutorials, or unreleased fixes are required.

For binary installation, install the MoveIt2 packages matching ROS 2 Humble.

For source installation, create a colcon workspace, import MoveIt2 repositories, install dependencies with rosdep, then build with colcon.

If dependency resolution is slow or unavailable, use a reachable rosdep source and update cache before rebuilding.

Launch a demo scene in RViz to verify planning plugins, robot model loading, and visualization.

# Project Workspace

Clone or copy this repository to the board.

From ros2_ws, source ROS 2 Humble and run rosdep install -r --from-paths src --ignore-src --rosdistro humble -y.

Build with colcon build --symlink-install.

Source install/setup.bash.

Launch the robot controller package and verify feedback topics before sending motion commands.

Run perception nodes one at a time, then verify output topics before starting the full task manager.

# Validation Checklist

Confirm both robot arms are reachable over the configured network.

Confirm RGB-D camera topics publish synchronized color, depth, and camera-info messages.

Confirm calibration files match the physical camera and tool mounting.

Confirm gripper open and close commands work at low force.

Confirm MoveIt2 planning and controller feedback before running the full box-opening sequence.